Reflection Report: Timer Peripheral Verification

1. What was Verified

The verification environment successfully achieved high functional coverage and protocol validation for the Timer Peripheral. The following key aspects were verified:

- Handshake Protocol: The REQ/GNT timing was monitored using SVA, ensuring the bus protocol's integrity.

- Register Operations: Randomized Read/Write transactions verified the accessibility and functionality of the Control, Load, Status, and Count registers.

- Operational Modes: Both One-Shot and Auto-Reload modes were exercised and validated through cross-coverage.

- Expiration Logic: The transition of the timer from a running state to an expired state was verified against the reference model.

2. Identified Issues (What the RTL Missed)

The verification process exposed several critical RTL bugs that would have caused system-level failures:

- Protocol Latency: A violation was caught where the GNT signal was asserted 4 cycles after REQ, exceeding the specification of 3 cycles.

- Status Clear Failure (W1C): A major bug was identified where writing a '1' to the Expired bit in the Status register (0x08) failed to clear the flag.

- Sticky Flag in Auto-Reload: In Auto-Reload mode, the Expired flag remained high indefinitely, preventing the software from detecting subsequent expiration events.

- Timing Skew: The expired_flag was found to be asserted one clock cycle late relative to the counter reaching zero.

3. What was Missed (Future Scope)

Due to project constraints, a few areas were not fully explored:

- Back-to-Back Transactions: Intense bursts of transactions without idle cycles between them were not fully stressed.

- Reset During Operation: While power-on reset was verified, triggering a reset in the middle of a countdown was not part of the current test suite.

- Register Aliasing: Tests for invalid or unmapped addresses were minimal.

4. Proposed Improvements

To enhance the robustness of this environment in a real-world scenario, I would implement:

- UVM Migration: Transitioning from native SystemVerilog to a full UVM (Universal Verification Methodology) architecture for better scalability and reporting.

- Register Abstraction Layer (RAL): Implementing a RAL model to handle register sequences more efficiently and track mirrored values automatically.

- Corner Case Sequences: Adding specific "Directed Tests" to target edge cases like reloading the timer exactly at the moment it expires.